Defender Dashboard Screen

Implementation Plan

Jaunt Motors

March 2026

ESP32-S3 + LVGL + CAN Bus

# Context

Building the software for a central dashboard display in a 2010 Land Rover Defender EV conversion. The display is a Waveshare ESP32-S3-Touch-LCD-4.3B-BOX (800×480, portrait orientation) showing a grid of ~24 warning light icons and a central gear selector (P/R/N/D/S). Data comes via CAN bus from the VCU and BMS.

The team has an existing prototype using ESP-IDF + SquareLine Studio. This plan defines a new, clean version with hand-coded LVGL and a data-driven architecture designed for team collaboration.

Hardware:

- ESP32-S3 (dual-core, 240 MHz, 16MB Flash, 8MB PSRAM)

- ST7701 RGB LCD (480×800 portrait via 90° rotation)

- GT911 capacitive touch controller

- CH422G I2C GPIO expander

- TWAI/CAN at 500 kbps (TX=GPIO15, RX=GPIO16)

# Project Structure

The project follows ESP-IDF conventions with clear module separation:

defender-dash-screen/

├── CMakeLists.txt

├── sdkconfig.defaults

├── partition_table/

│   └── partition-table.csv

├── lib/

│   ├── CH422G/                  (Reused from existing)

│   └── Config/                  (Reused from existing)

└── main/

├── CMakeLists.txt

├── Kconfig.projbuild

├── idf_component.yml

├── main.c                   App entry: init LCD, CAN, UI

├── lvgl_port.c/h            Reused (display driver)

├── waveshare_rgb_lcd_port.c/h  Reused (LCD hardware)

├── can/

│   ├── can_bus.c            CAN RX task + message dispatch

│   └── can_bus.h

├── ui/

│   ├── ui_main.c/h          Screen layout, 100ms update timer

│   ├── ui_warning_grid.c/h  Data-driven icon grid

│   ├── ui_gear_display.c/h  Central gear letter

│   └── ui_styles.c/h        Shared colors, fonts

├── data/

│   └── vehicle_state.c/h    CAN-to-UI bridge, thread-safe

└── icons/

├── icon_defs.c/h        Master icon config table

├── icons.h              LV_IMG_DECLARE for all icons

└── img_*.c              One C array per icon (~24 files)

# Key Architecture: Data-Driven Icon Table

All icon configuration lives in a single table. Adding or changing icons requires editing only this table plus adding an image file. No other code changes needed.

// icon_defs.h

typedef struct {

icon_id_t           id;

const char          *name;        // Debug name

const lv_img_dsc_t  *img;         // LVGL image

icon_color_t        color;        // RED, AMBER, BLUE, GREEN

uint8_t             grid_row;     // Grid position

uint8_t             grid_col;

uint32_t            can_msg_id;   // 0 = not yet mapped

uint8_t             can_byte;     // Byte index in CAN data

uint8_t             can_bit_mask; // Bit mask

} icon_def_t;

CAN mappings are all placeholder (0x000) until the team maps them from VCU/BMS specs. To add a new warning light:

- Export icon from Figma as white-on-transparent PNG, convert to LVGL C array

- Add entry to icon_id_t enum in icon_defs.h

- Add one row to icon_defs[] table with image, color, grid position, and CAN mapping

# CAN Architecture

The CAN bus and LVGL UI run on separate ESP32-S3 cores for optimal performance:

CAN Bus (TWAI) → can_bus.c RX Task (Core 0)

→ Dispatch by msg_id to registered handlers

→ vehicle_state.c updates state (mutex-protected)

→ ui_main.c LVGL timer (100ms, Core 1) polls state

→ ui_warning_grid / ui_gear_display update widgets

The UI polls vehicle state every 100ms, which is sufficient for warning light response times. The vehicle_state module provides thread-safe access via FreeRTOS mutex.

# Icon Rendering

- Icons exported as white on transparent PNGs, converted to LVGL C arrays

- LVGL img_recolor applies the correct color (red/amber/blue/green) at runtime

- OFF state: 20% opacity (dim silhouette visible)

- ON state: 100% opacity with designated color

- BLINK state: LVGL timer toggles opacity

# What We Reuse vs Write Fresh

## Reused from Existing Project

- lvgl_port.c/h — working display driver with rotation

- waveshare_rgb_lcd_port.c/h — LCD hardware init

- lib/CH422G/ — GPIO expander driver

- lib/Config/ — I2C master init

- partition_table/partition-table.csv

- sdkconfig.defaults (with cleanup)

- Kconfig.projbuild, idf_component.yml

## Written Fresh

- main.c — simplified init (no factory test loop)

- can/ — proper CAN abstraction replacing basic waveshare_twai_port

- ui/ — all hand-coded LVGL replacing SquareLine Studio output

- data/vehicle_state.c/h — new CAN-to-UI bridge

- icons/ — new image assets from Figma + icon definitions table

# Implementation Phases

## Phase 1: Project Scaffold

- Create directory structure

- Copy reusable hardware driver files from existing project

- Write CMakeLists.txt files and build config

- Write minimal main.c that boots LCD with a black screen

- Verify it compiles

## Phase 2: UI Framework

- Create ui_styles.c/h with dashboard color definitions

- Create ui_main.c/h with layout containers (top grid, center gear, bottom grid)

- Create icon_defs.c/h with enum and full table for all 24 icons (placeholder images initially)

- Create ui_warning_grid.c/h — reads icon_defs table, creates lv_img objects in grid

- Create ui_gear_display.c/h — large center label (Montserrat 48 initially, Eurostile later)

- Create placeholder icon images so UI renders

## Phase 3: CAN Bus Layer

- Write can_bus.c/h — TWAI init, FreeRTOS RX task, handler registration API

- Write vehicle_state.c/h — state struct, mutex, auto-registers CAN handlers from icon_defs

- Wire up 100ms LVGL timer to poll vehicle_state and update UI

## Phase 4: Icon Assets

- Download all ~24 icon images from Figma design (white on transparent)

- Convert to LVGL C arrays (CF_TRUE_COLOR_ALPHA format)

- Replace placeholder images with real icons

- Fine-tune grid positions to match Figma layout

# Verification

Build check:

idf.py build completes without errors.

Visual check:

Flash to device, verify black screen with icon grid and gear letter displayed correctly.

CAN check:

Send test CAN messages via bus simulator, verify icons toggle on/off and gear selector updates.

Team check:

Another team member can add a new icon by editing only icon_defs.c + adding one image file.

# Files to Modify/Create

Approximately 25 files total:

| File | Action |
| --- | --- |
| CMakeLists.txt | Create |
| sdkconfig.defaults | Copy + clean |
| partition_table/partition-table.csv | Copy |
| lib/CH422G/* | Copy |
| lib/Config/* | Copy |
| main/CMakeLists.txt | Create |
| main/Kconfig.projbuild | Copy + adapt |
| main/idf_component.yml | Copy |
| main/main.c | Create (fresh) |
| main/lvgl_port.c/h | Copy |
| main/waveshare_rgb_lcd_port.c/h | Copy |
| main/can/can_bus.c/h | Create |
| main/ui/ui_main.c/h | Create |
| main/ui/ui_warning_grid.c/h | Create |
| main/ui/ui_gear_display.c/h | Create |
| main/ui/ui_styles.c/h | Create |
| main/data/vehicle_state.c/h | Create |
| main/icons/icon_defs.c/h | Create |
| main/icons/icons.h | Create |
| main/icons/img_*.c | Create (~24 files) |